Dear Fellow ECoG Researcher,

These are a series of motor tasks of varying complexity collected together under the heading “gestures”. They are meant to test interactions between primary motor cortex and other motor areas. These data are not yet published.

Please keep in mind that these anonymized data are from real patients who donated time in a difficult period of their lives to advance our understanding of the brain. Any publication involving these data MUST include the following in the methods section of the manuscript, without modification:

Ethics statement: All patients participated in a purely voluntary manner, after providing informed written consent, under experimental protocols approved by the Institutional Review Board of the University of Washington (#12193). All patient data was anonymized according to IRB protocol, in accordance with HIPAA mandate. It was made available through the library described in “A Library of Human Electrocorticographic Data and Analyses” by Kai Miller [Reference], freely available at

https://searchworks.stanford.edu/view/zk881ps0522

There are an array of tasks for each patient, each of which is contained in a folder data/##, where ## denotes the 2 letter patient code.

Task files of all types have the following variables:

"data" (time x number of channels): These are the data.

- sampled at 1000Hz

- scale factor: 1 amplifier unit = .0298 microvolts

- built-in band pass 0.15 to 200 Hz,

- but a 1 pole band pass, so there is no sharp corner at 200Hz.

-The amplitude roll-off function is in the file “ns_1k_1_300_filt.mat”

"dg" (time x number of fingers): These are the finger flexions, measured from the hand contralateral to grid placement. Note that they are in the raw form input. They are in 40ms blocks, and have non-zero offset. The 5 columns correspond to the digits thumb-index-middle-ring-little in order. Finger positions were recorded using a 5 degree-of-freedom dataglove sensor (5 dt, Irvine, CA).

"srate": Sampling rate. 1000Hz

"stim" (time x 1): Screen cue. This is the cue on the screen at each point in time (note that this is different than the timing of actual behavior, which will be revealed by the finger flexion positions, and different for each patient).

"stimtext": cell array, containing text corresponding to numerical value in “stim”. Sometimes these values are repeated, so that they will be cued at double frequency.

The individual task files are as follows:

- Baseline – “##_base.mat”: The basic baseline fixation task was performed by the subjects fixating with their eyes open on an ‘‘X’’, on the wall 3 m away, for several (2-3) minutes.

"stim" (time x 1): all zeros.

- Finger flexion – “##_fingerflex.mat”: finger movement task, subjects were cued with a word displayed on a bedside monitor indicating which finger to move during 2- second movement trials. The subject performed self-paced movements in response to each of these cues, and they typically moved each finger 2–5 times during each trial, but some trials included many more movements.

"stim" (time x 1):

0 – Inter-stimulus interval

1 – thumb

2 – index finger

3 – middle finger

4 – ring finger

5 – little finger

- Thumb and forefinger flexion  – “##_thumbfore.mat”: Thumb and index finger flexion when subjects were cued with a word displayed on a bedside monitor indicating which finger to move.

"stim" (time x 1):

0 – Inter-stimulus interval

1 – thumb

2 – index finger

- Pinching – “##_pinch.mat”: pinching when subjects were cued with a word displayed on a bedside monitor.

"stim" (time x 1):

0 – Inter-stimulus interval

1 – pinch

- Self-determined gestures – “##_freeform.mat”: Subjects were instructed to make a compound gesture of their choosing, involving more than one finger, when cued to do so.

"stim" (time x 1):

0 – Inter-stimulus interval (rest)

1 – make gesture

- Right-hand & Left-hand flexion – “##_rh_lh.mat”:

"stim" (time x 1):

0 – Inter-stimulus interval (rest)

1 – right fist close/open

2 – left fist close/open

3 – close/open both fists

- Hand flexion & tongue extension – “##_mot_TH.mat”: cue-based movement of the hand and tongue.

"stim" (time x 1):

0 – Inter-stimulus interval (rest)

1 – tongue movement

2 – hand movement (fist close/open)

- Glove fingers, pinch, and grasp – “##_glovefingersgrasp.mat”: finger movement, pinching, and open/close fist (grasp) task when subjects were cued with a word displayed on a bedside monitor indicating which finger / pinch / grasp to move.

"stim" (time x 1):

0 – Inter-stimulus interval

1 – thumb

2 – index finger

3 – middle finger

4 – ring finger

5 – little finger

6,8 – pinch thumb and forefinger together

7,9 – fist close/open

| Subject | BP | CA | CC | DE | WM |
| --- | --- | --- | --- | --- | --- |
| Age | 18 |  | 21 |  | 19 |
| Sex | F |  | M |  | F |
| Handedness | R |  | R |  | R |
| Base | y | y | y | y | y |
| Fingers | y | y |  | y | y |
| Thumbfore |  |  | y |  | y |
| Pinch | y | y | y |  | y |
| Freeform | y | y | y |  | y |
| Grasp |  |  |  | y |  |
| RH-LH | y |  |  |  |  |
| motTH |  | y |  |  |  |
| GlovefingersGrasp |  |  |  | y |  |
| Localize method | XRMR, LOC | CTMR, LOC | CTMR | LOC | XRMR |

Electrode locations were localized depending on the imaging available:

- If an MRI was available, patient brain surfaces were rendered, and electrodes were localized with either CT or X-Ray. In some cases, MRIs were sliced thickly (~6mm), so surface quality was not good. This MRI-CT localization approach (CTMR) is described in “Automated electrocorticographic electrode localization on individually rendered brain surfaces” by D Hermes, et al in Journal of Neuroscience Methods, 2009. The MRI-X-Ray localization approach (XRMR) is described in “Brain surface electrode co-registration using MRI and x-ray.” by Miller, et al in IEEE Eng Med Biol Soc, 2010. The individualized MRI anatomy files (for BP, CA, CC, WM) are in the folder brains/, with files named – “##_brain.mat”, with variables:

  - "brain": This is a structure representing the tessellated brain surface. It can easily be plotted with the CTMR package (see “Automated electrocorticographic electrode localization on individually rendered brain surfaces” by D Hermes, et al in Journal of Neuroscience Methods, 2009)

  - "locs" (number of channels x 3): Electrode locations, for plotting on the rendered brain.

- If an MRI was not available (DE), and for 2 others (BP, CA), anatomy was determined by x-ray alone (LOC), for plotting on a standardized brain. For each, there is a file titled “##_locs.mat” in the folder locs/, and each file contains a single variable:

  - "locs" (number of channels x 3): Electrode locations, in Taliarach coordinates, for plotting on the standardized brain. These were obtained using the LOC package, and can be plotted with it as well (code in “loc” folder). From “Cortical electrode localization from X-rays and simple mapping for electrocorticographic research: The “Location on Cortex” (LOC) package for MATLAB” in J Neurosci Meth, 2007.

Some things to consider for these data:

- Note that electrode montages may be different from same task in same patient elsewhere in this library.

- I have not rejected bad channels from these data.

- For patient “BP”, bipolar electromyography was recorded from the contralateral limb, and is contained in the variables “EMG_FlexorCarpi” and “EMG_ExtensorCarpi”.

Best Wishes!

Kai Miller,

Stanford University, 2015 (revised 2018)

kai.miller@stanford.edu, miller.kai@mayo.edu