Dear Fellow ECoG Researcher,

These are mouse-tracking data recorded at Harborview Hospital, in Seattle, WA.

Please keep in mind that these anonymized data are from real patients who donated time in a difficult period of their lives to advance our understanding of the brain. Any publication involving these data MUST include the following in the methods section of the manuscript, without modification:

Ethics statement: All patients participated in a purely voluntary manner, after providing informed written consent, under experimental protocols approved by the Institutional Review Board of the University of Washington (#12193). All patient data was anonymized according to IRB protocol, in accordance with HIPAA mandate. It was made available through the library described in “A Library of Human Electrocorticographic Data and Analyses” by Kai Miller [Reference], freely available at https://searchworks.stanford.edu/view/zk881ps0522

During the study, each patient was in a semi-recumbent position in a hospital bed about 1 m from a computer monitor. The patient used a mouse on a mousepad to use a white cursor track a green target moving counter-clockwise in a circle of diameter ~85% of the monitor height, corresponding to ~10-15cm diameter tracing. The hand used to control the mouse was contralateral to the implanted electrode array.

| Subject |  | Age | Sex | Hand | Grid Location | Seizure Focus |
| --- | --- | --- | --- | --- | --- | --- |
| FP |  | 23 | M | R | L Frontotemporal | L Temporal |
| GC |  | 24 | M | R | L Anterior Frontotemporal | L Anterior Frontal |
| RH |  | 38 | M | R | R Frontal | R Frontal |
| RR |  |  |  |  |  |  |

Each datafile has 7 variables:

- "subject": This is the 2-letter patient code

- "data" (time x number of channels): These are the data. Sampled at 1000Hz, with a built-in bandpass 0.15-200 Hz: 1 pole band pass, so there is no sharp corner at 200Hz. The amplitude roll-off function is in the file “ns_1k_1_300_filt.mat”

- “TargetPosX” (time x 1): x-axis position of target

- “TargetPosY” (time x 1): y-axis position of target

- “CursorPosX” (time x 1): x-axis position of joystick-controlled cursor

- “CursorPosY” (time x 1): y-axis position of joystick-controlled cursor

- "electrodes" (number of channels x 3): Electrode locations, in Taliarach coordinates, for plotting on the standardized brain. These were obtained using the LOC package, and can be plotted with it as well (code in “loc” folder). From “Cortical electrode localization from X-rays and simple mapping for electrocorticographic research: The “Location on Cortex” (LOC) package for MATLAB” in J Neurosci Meth, 2007.

Best Wishes!

Kai Miller, Stanford University, 2015 (revised 2018)

kai.miller@stanford.edu, miller.kai@mayo.edu